DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

CSA1203 – Computer Architecture

Design and Analysis of an Energy-Efficient Processor

and Hierarchical Memory System

Assignment 1 — Full Report

| Field | Details |
| --- | --- |
| Course Code & Name | CSA1203 – Computer Architecture |
| Course Outcomes Covered | CO1, CO3, CO4 |
| Bloom's Taxonomy Level | L3 – Apply ; L4 – Analyze |
| SDG Mapping | SDG 7 (Affordable & Clean Energy), SDG 9 (Industry, Innovation & Infrastructure), SDG 12 (Responsible Consumption & Production) |
| Student Name | Siva Balan K |
| Register / Roll Number | 192572037 |
| Date of Submission | 01/09/2026 |

# Problem Statement

A smart manufacturing company plans to develop an energy-efficient processor for an industrial monitoring system. The processor must efficiently handle continuous sensor data streams and real-time control/actuation requirements while minimising power consumption. This report analyses the required functional units, instruction execution flow, pipelined datapath, hazard-handling strategy, and hierarchical memory system, and proposes an optimised processor and memory organisation supporting the target SDGs.

# 1. Functional Units, Instruction Execution, Addressing Modes & Performance Analysis

## 1.1 System Architecture — Functional Units

The industrial monitoring processor is organised around five functional blocks connected by a shared system bus, as shown in Figure 1.

| CPU ALU + Control Unit + Register File | Cache (L1/L2/L3) On-chip hierarchy | Main Memory DRAM | Secondary Storage NAND Flash |
| --- | --- | --- | --- |
| SYSTEM BUS (Address / Data / Control) |
| I/O Controller DMA + Interrupt Ctrl | Sensor Interface ADC / Digital I/O | Actuator Interface PWM / Control Out | Comm. Module UART / CAN / Ethernet |

Figure 1: Functional-unit block diagram of the proposed industrial monitoring processor

### Component Roles

| Unit | Function | Design Consideration |
| --- | --- | --- |
| CPU (ALU + CU + Registers) | Executes arithmetic/logic operations, controls instruction flow, holds operands | Low-power in-order 5-stage RISC core with clock gating |
| Cache Hierarchy (L1/L2/L3) | Reduces average memory access latency | Split L1 I/D cache; unified L2/L3 (see Section 3) |
| Main Memory (DRAM) | Stores program and sensor-data buffers | Low-power DDR with self-refresh mode |
| Secondary Storage (NAND Flash) | Non-volatile event/data logging | Wear-levelled flash controller |
| I/O Controller (DMA + Interrupts) | Moves sensor data without CPU intervention; services interrupts | Burst-mode DMA to cut CPU wake-ups |
| System Bus | Shared address/data/control path between all units | Split-transaction bus to reduce structural stalls |

## 1.2 Instruction Execution Cycle

Every instruction in the processor follows the classic fetch–decode–execute cycle, extended here into five discrete stages that map directly onto the pipeline discussed in Section 2:

- Instruction Fetch (IF): Instruction is read from I-cache using the Program Counter (PC); PC is incremented.

- Instruction Decode (ID): Opcode and operands are decoded; register file is read; control signals generated.

- Execute (EX): ALU performs the arithmetic/logic operation, computes branch targets, or calculates effective memory addresses.

- Memory Access (MEM): Load/store instructions access D-cache; other instructions pass through unchanged.

- Write Back (WB): Result is written back into the register file.

Example — reading a sensor threshold comparison instruction "CMP R1, R2" followed by a conditional branch: IF fetches CMP, ID reads R1/R2, EX computes R1−R2 and sets condition flags, MEM is unused, WB updates the flags register.

## 1.3 Addressing Modes Used

| Addressing Mode | Example | Typical Use in this System |
| --- | --- | --- |
| Immediate | MOV R1, #75 | Loading a fixed sensor threshold (e.g., 75°C alarm limit) |
| Register | ADD R3, R1, R2 | Fast ALU operations on values already in registers |
| Direct (Absolute) | LOAD R4, [0x2000] | Reading a memory-mapped I/O status register |
| Register Indirect | LOAD R5, [R6] | Accessing a sensor buffer via a pointer register |
| Indexed | LOAD R7, [R8 + R9] | Walking through an array of periodic sensor samples |
| Base + Displacement | LOAD R10, 12(R11) | Accessing a specific field inside a sensor-data structure |

## 1.4 Performance Analysis — CPI, Clock Cycles, Execution Time

Assume a target clock rate of 1.2 GHz and the following instruction mix, obtained by profiling a representative industrial-monitoring workload (sampling sensors, filtering, threshold checks, and actuation):

| Instruction Class | Frequency | Cycles per Instruction (CPI) | Weighted CPI Contribution |
| --- | --- | --- | --- |
| ALU / Logic | 45% | 1 | 0.45 |
| Load / Store | 25% | 3 | 0.75 |
| Branch | 15% | 2 | 0.30 |
| Sensor I/O | 10% | 4 | 0.40 |
| Multiply / Divide | 5% | 5 | 0.25 |

Average CPI:

CPI = Σ (Frequency_i × CPI_i) = 0.45+0.75+0.30+0.40+0.25 = 2.15 cycles/instruction

Clock cycle time:

T_clk = 1 / 1.2 GHz = 0.833 ns

Execution time for a program of 1,000,000 (10⁶) instructions:

Execution Time = Instruction Count × CPI × T_clk

= 10⁶ × 2.15 × 0.833 ns ≈ 1.791 × 10⁶ ns ≈ 1.79 ms

Processor throughput (MIPS):

MIPS = Clock Rate (MHz) / (CPI × 10⁶) = 1200 / 2.15 ≈ 558 MIPS

Benchmarking note: For an embedded industrial controller, EEMBC AutoBench / CoreMark-style kernels (filtering, control loops, checksum) are more representative than SPEC CPU, since they mirror sensor-processing and control-loop behaviour rather than general desktop workloads. Reported CoreMark/MHz is therefore used, in addition to raw MIPS, as the benchmarking figure of merit for this design.

# 2. Pipelined Processor Organization & Hazard Analysis

## 2.1 Pipelined Datapath

A 5-stage in-order pipeline (IF – ID – EX – MEM – WB) is adopted to raise instruction throughput without a large area/power penalty, matching the CPI targets above. Table 2 traces four instructions through the pipeline, showing a data hazard (I2, I3 depend on I1) and a control hazard (I4, a branch).

| Instruction | T1 | T2 | T3 | T4 | T5 | T6 | T7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I1: LOAD R1,[R2] | IF | ID | EX | MEM | WB |  |  |
| I2: ADD R3,R1,R4 |  | IF | ID* | EX | MEM | WB |  |
| I3: SUB R5,R3,R6 |  |  | IF | ID* | EX | MEM | WB |
| I4: BR Loop |  |  |  | IF | (stall) | IF' | ID |

Table: Pipeline execution trace (ID* = decode stalled/forwarded due to hazard; IF' = re-fetch after branch resolution)

## 2.2 Hazard Types Identified

| Hazard Type | Cause in this Design | Example |
| --- | --- | --- |
| Structural Hazard | A single unified memory port is shared by the IF stage (instruction fetch) and the MEM stage (data access) in the same cycle | I1 in MEM and a later instruction's IF collide on the shared cache port |
| Data Hazard (RAW) | An instruction needs a register value that a previous, still-in-flight instruction has not yet written back | I2 (ADD R3,R1,R4) needs R1 loaded by I1 (LOAD R1,[R2]) two cycles earlier |
| Control Hazard | The next instruction to fetch is unknown until a branch is resolved in EX | I4 (BR Loop) — instructions after it cannot be safely fetched until the branch outcome is known |

## 2.3 Hazard-Mitigation Techniques Applied

- Forwarding / Bypassing: The EX/MEM and MEM/WB pipeline registers feed results directly back to the EX stage input, so I2 can use I1's loaded value the cycle it becomes available instead of waiting for WB — this removes most RAW stalls for ALU-to-ALU dependencies.

- Stalling (Pipeline Interlock): When a load result is needed by the immediately following instruction (load-use hazard), a single bubble is inserted, since a plain load cannot be forwarded one cycle early.

- Structural resolution: Split I-cache and D-cache ports remove the fetch/memory-access bus conflict entirely, converting the structural hazard into a non-issue for the common case.

- Branch Prediction: A 2-bit saturating-counter dynamic predictor is used for the frequent threshold-check branches in the sensor-monitoring loop; on misprediction the 2 fetched instructions are squashed and the pipeline restarts from the correct target (as shown by IF' in the trace).

## 2.4 Superscalar, Out-of-Order and Speculative Execution — Comparison

| Technique | Principle | Benefit for this Workload | Cost / Risk |
| --- | --- | --- | --- |
| Superscalar (2-way) | Fetch/decode/issue ≥2 instructions per cycle to duplicated execution units | Exploits independent ALU + load/store pairs common in filtering code | Extra hardware (decoders, issue logic); diminishing returns if ILP is low |
| Out-of-Order Execution | Instructions issue as soon as operands are ready, not strictly in program order, using a reorder buffer | Hides load latency from sensor buffers behind independent ALU work | Complex scheduling logic, larger register file, higher power — a concern for an energy-constrained design |
| Speculative Execution | Predicted branch path is executed before the branch resolves; results committed only if correct | Keeps the pipeline full across the frequent conditional threshold checks | Wasted energy on squashed instructions; requires safe rollback for I/O side-effects |

## 2.5 Suitability of Simultaneous Multithreading (SMT)

The industrial monitoring workload is characterised by frequent, short waits for sensor I/O and memory (DMA transfers, ADC conversions), during which a single-threaded pipeline sits idle. SMT allows a second hardware thread (e.g., a diagnostics/logging task) to issue instructions into the otherwise-empty pipeline slots during these stalls, raising functional-unit utilisation without duplicating the full core. Given the design's power budget, 2-way SMT is recommended rather than deep out-of-order plus 4-way SMT: it captures most of the latency-hiding benefit for I/O-bound stalls at a much smaller area/power cost, which better matches the CO3 goal of "improving processor utilisation" under the energy constraints of Section 4.

# 3. Hierarchical Memory System & Performance Evaluation

## 3.1 Cache Hierarchy Design

| Level | Type / Size | Access Latency | Local Hit Rate | Role |
| --- | --- | --- | --- | --- |
| L1 (I + D, split) | 32 KB each, 4-way | 1 cycle | 95% | Serve the tight sensor-sampling / control loop instructions and hot data |
| L2 (unified) | 256 KB, 8-way | 10 cycles | 90% (of L1 misses) | Buffer larger filter windows and multi-sensor working sets |
| L3 (shared) | 2 MB, 16-way | 30 cycles | 98% (of L2 misses) | Shared across cores/threads (incl. SMT) to cut off-chip traffic |
| Main Memory (DRAM) | 512 MB – 1 GB | 100 cycles | — | Program image, large sensor-history buffers |

## 3.2 Average Memory Access Time (AMAT)

AMAT = HitTime_L1 + MissRate_L1 × ( HitTime_L2 + MissRate_L2 × ( HitTime_L3 + MissRate_L3 × Time_Mem ) )

Substituting the values from Table above (miss rate = 1 − hit rate):

AMAT = 1 + 0.05 × ( 10 + 0.10 × ( 30 + 0.02 × 100 ) )

= 1 + 0.05 × ( 10 + 0.10 × 32 )

= 1 + 0.05 × ( 10 + 3.2 )  =  1 + 0.05 × 13.2

AMAT ≈ 1.66 cycles

At 1.2 GHz this corresponds to roughly 1.38 ns average memory access time — well within the latency budget needed to keep the 5-stage pipeline's MEM stage from stalling on the common case, since 95% of accesses resolve in a single cycle at L1.

## 3.3 Virtual Memory with Paging

- Page size: 4 KB, mapped through a 2-level page table to keep page-table memory overhead small on an embedded budget.

- TLB: 64-entry, fully associative, translating the hot working set (sensor buffers, control-loop code) in parallel with the L1 access to avoid adding translation latency to the common case.

- Protection: Paging isolates the real-time control tasks from lower-priority logging/diagnostic tasks (relevant if SMT runs a second thread), preventing a runaway logging routine from corrupting control-loop memory.

- Page faults: Handled by a lightweight fault handler; since real-time deadlines matter, critical control-loop pages are pinned (non-swappable) while logging/history buffers may be paged to flash.

## 3.4 Memory Technology Comparison

| Technology | Latency | Bandwidth | Volatility | Endurance | Power | Best Role Here |
| --- | --- | --- | --- | --- | --- | --- |
| SRAM | Sub-ns – ~1 cycle | Very High | Volatile | Unlimited | Low static, high density-cost | L1/L2/L3 cache cells |
| DRAM | ~50–100 cycles | High | Volatile (needs refresh) | Unlimited | Moderate (refresh power) | Main memory |
| NAND Flash | µs-level (block) | Moderate | Non-volatile | ~10⁴–10⁵ writes/cell | Low (idle), higher on write | Event/data logging, firmware storage |
| MRAM | ~10s of ns | Moderate–High | Non-volatile | Very high (~10¹²) | Very low standby, no refresh | Non-volatile buffer for critical sensor state (survives power loss) |
| RRAM | ~10s of ns | Moderate | Non-volatile | High (~10⁶–10⁹) | Very low | Emerging option for compact non-volatile working memory / edge-AI weights |

For this design, SRAM/DRAM form the performance-critical volatile hierarchy (as sized above), while MRAM is proposed as a small non-volatile shadow buffer for the last known sensor/actuator state — allowing the controller to resume safely after a power interruption without waiting on the slower NAND flash log, directly supporting energy-resilient, dependable operation.

# 4. Results, Comparison & Engineering Recommendation

## 4.1 Summary of Computed Results

| Metric | Value |
| --- | --- |
| Weighted Average CPI | 2.15 cycles/instruction |
| Clock Rate | 1.2 GHz (T_clk = 0.833 ns) |
| Execution Time (10⁶ instructions) | ≈ 1.79 ms |
| Throughput | ≈ 558 MIPS |
| Pipeline Depth | 5 stages (IF-ID-EX-MEM-WB) |
| AMAT (memory hierarchy) | ≈ 1.66 cycles (≈ 1.38 ns @ 1.2 GHz) |
| L1 / L2 / L3 hit rates | 95% / 90% / 98% |

## 4.2 Bottleneck Analysis

- Load/Store instructions carry the highest per-instruction CPI (3) and the highest frequency after ALU ops (25%), making the memory system — not raw ALU throughput — the dominant performance bottleneck; this justifies the 3-level cache investment in Section 3 over, e.g., a deeper ALU pipeline.

- Control hazards from frequent threshold-check branches (15% of the mix) would cost roughly 2 wasted cycles per misprediction without prediction; the 2-bit dynamic predictor keeps this cost low for the mostly-repetitive sensor-polling branches.

- Sensor I/O (10% of the mix, CPI 4) is well suited to latency-hiding via SMT rather than deeper speculation, since these are genuine external waits, not computation.

## 4.3 Engineering Recommendation

Based on the above analysis, the recommended design point is:

- A 5-stage in-order pipeline with forwarding and single-cycle load-use stalling, extended to 2-way superscalar issue for independent ALU/load pairs — full out-of-order execution is not recommended, as its power/complexity cost outweighs the ILP available in this control-loop-dominated workload.

- A 2-bit dynamic branch predictor for the frequent, repetitive threshold-check branches, combined with speculative fetch limited to a shallow (2-instruction) window to cap misprediction energy waste.

- 2-way SMT to hide sensor-I/O and DMA-wait latency using a secondary logging/diagnostics thread, improving functional-unit utilisation without a second full core.

- A 3-level SRAM cache hierarchy (32 KB/32 KB L1, 256 KB L2, 2 MB L3) sized to reach ~1.66-cycle AMAT, backed by low-power DRAM main memory with self-refresh during idle periods.

- An MRAM non-volatile state buffer for critical sensor/actuator values, with NAND flash for bulk event logging — trading a small area cost for power-loss resilience without the write-endurance concerns of using flash directly for hot state.

- Dynamic Voltage and Frequency Scaling (DVFS), clock-gating idle pipeline stages, and DRAM self-refresh during predictable low-activity monitoring intervals to minimise energy per operation.

## 4.4 Tool / Simulation Evidence

To validate the figures derived analytically in Sections 1.4, 2.1 and 3.2, a supporting Python script (processor_simulator.py) was written and executed. It independently (a) computes the weighted CPI, execution time, and MIPS from the profiled instruction mix, (b) computes the multi-level AMAT from the cache hit-rate/latency table, and (c) simulates the 5-stage pipeline trace of Section 2.1 cycle-by-cycle, automatically detecting the load-use hazard and propagating its backpressure to subsequent instructions. The script's console output, captured directly from the run, is shown in Figures 2–6 below.

Figure 2: Script output — CPI, execution time, and throughput calculator (Section 1.4 verification)

Figure 3: Script output — multi-level AMAT calculator (Section 3.2 verification)

Figure 4: Script output — 5-stage pipeline hazard trace (Section 2.1 verification)

Figure 5: Script output — hazard-detection summary and simulation completion

Figure 6: Confirmation of script execution (run environment)

The script's output matches the hand-derived values exactly: weighted CPI = 2.1500, execution time ≈ 1.7917 ms, throughput ≈ 558.14 MIPS, and AMAT ≈ 1.6600 cycles. The pipeline simulator additionally confirms that the single load-use stall on I2 (marked ** at cycle T4) automatically delays I3 and I4 by one cycle each (marked 'held'), which is the correct backpressure behaviour of a real in-order pipeline and matches the hazard-mitigation discussion in Section 2.3.

Full source code for processor_simulator.py, along with this captured output, is included in the project's GitHub repository (see Deliverables Checklist).

# 5. Reflection: Design Decisions, SDG Relevance & Learning Outcomes  (10 Marks)

## 5.1 Justification of Key Design Decisions

The design deliberately favours a moderate-complexity in-order/2-way-superscalar pipeline with SMT over a full out-of-order superscalar core. This trade-off was chosen because the target application — industrial sensor monitoring — has a fairly predictable, loop-dominated instruction stream with limited instruction-level parallelism, so the large area and power overhead of full OoO hardware (reservation stations, large reorder buffers) would not be repaid by a proportional performance gain. Similarly, the 3-level cache hierarchy was sized around the measured Load/Store frequency (25%) rather than a generic desktop profile, and MRAM was introduced specifically to solve the power-loss resilience requirement of an industrial controller, which a purely SRAM/DRAM/Flash system would not address as efficiently.

## 5.2 Connection to Sustainable Development Goals

| SDG | How the Design Supports It |
| --- | --- |
| SDG 7 — Affordable and Clean Energy | DVFS, clock-gating, DRAM self-refresh, and a right-sized (not over-provisioned) pipeline and cache hierarchy reduce energy per operation, lowering the electrical load the monitoring system places on the plant's power system. |
| SDG 9 — Industry, Innovation and Infrastructure | The processor directly enables reliable, real-time industrial monitoring and control — core infrastructure for a smart-manufacturing facility — while the SMT and hazard-mitigation techniques represent applied innovation in processor micro-architecture for embedded industry use. |
| SDG 12 — Responsible Consumption and Production | Non-volatile MRAM state retention avoids re-running or discarding in-progress production data after power interruptions, and wear-aware NAND flash logging reduces premature hardware replacement — both cutting waste in the manufacturing process the processor monitors. |

## 5.3 Challenges Encountered

- Balancing performance against the energy budget: several individually attractive techniques (full OoO execution, deep speculation, 4-way SMT) had to be scaled back once their power cost was weighed against the modest ILP available in the target workload.

- Reasoning about hazards precisely: constructing the pipeline trace (Section 2.1) required carefully tracking which register values were available in which cycle to correctly identify where forwarding was sufficient versus where a stall was unavoidable.

- Choosing representative benchmark figures: general-purpose benchmarks (e.g., SPEC) do not reflect a sensor-driven control workload, so an EEMBC/CoreMark-style reference had to be justified instead.

## 5.4 Learning Outcomes and Possible Improvements

This assignment reinforced how the CPI/clock-time/execution-time equations, hazard analysis, and the multi-level AMAT formula are not independent topics but a single connected design process: a decision in one layer (e.g., adding SMT) changes the assumptions in another (e.g., cache contention, hazard frequency). A possible improvement for a future iteration would be to validate the analytical figures in Section 4 against a cycle-accurate simulator run (e.g., gem5) with the exact instruction mix used here, and to explore adaptive DVFS policies that respond to real-time sensor load rather than fixed idle-period gating.

GitHub Repository Link: https://github.com/Naveen7-ai/CSA-1203

# References

1. Hennessy, J. L., & Patterson, D. A. Computer Architecture: A Quantitative Approach.

2. Patterson, D. A., & Hennessy, J. L. Computer Organization and Design: The Hardware/Software Interface.

3. Course materials, CSA1203 – Computer Architecture.

4. United Nations Sustainable Development Goals — https://sdgs.un.org/goals